Computer Architecture LAB 2 By Jahangir Ikram

Experiment: To study the basic instruction processing in MIPS, basics of pipelining and problems in pipelining.

NOTES: 1. In today’s lab we will concentrate on instructions other than Branch instructions. 2. Please follow the handout exactly as you are guided to do so.

EXPERIMENT 1: Basic Instruction Processing the ADD instruction:

Step 1. Run the visual MIPS simulator by downloading it from my website from the following link. http://suraj.lums.edu.pk/~jikram.

Run the simulator. In the Architecture menu select MIPS 5-STAGE INTEGER PIPELINE. You will see the following screen. Now go to File Menu and select write

program. You will be asked to Enter a Starting Value of Program Memory Address where you want to write you program.	Enter any value in Decimal, eg, 67676 or 23424324, or something small and simple like 2000 and press OK. MIPS Integer Program Writer screen will appear. Here you can write a maximum of 25 instruction program which is more than enough for this course. By default all instruction are set to NOP which is translated as SLL R0,R0,R0. Fill exercise 1 at the end of this handout.

Step 2. Write a single instruction program ADD R5, R4, R3. Now press Assemble. The program appears on the side of the simulator. Save this program as SingleADD, just in case.

Step 3. The Instruction Fetch STEP.

Press Clock it to step through this program. As described in the class yesterday, the Program Counter (PC) output goes

to instruction memory (IM) and Instruction appears at the output of IM. Click on the instruction, an info bar appears. Fill exercise 2 at the end using the handout. Note that PC+4 is also calculated as next address.

Step 4: The Instruction Decode and Register Fetch Stage.

Press clock it again to go to the next step. For time being just ignore what changes have happened back in the IF stage. As you remember from the class, at this stage R4 as Rs and R3 as Rt go to the Register File (Regs).	BUT WHAT VALUES REGISTER FILE WILL RETURN? In a real program some useful info is suppose to be in Regs. The simulator asks you to initialize these two register with some initial values. Give R4 as 100 and R3 as 200. The inputs are done in Decimal. Note that these value appear somewhere on the top also. Also you can see register file contents anytime by clock Register Menu at the top. Another thing to be noted is that Opcode goes to Control Unit and it generates a lot of controls for rest of the instruction steps. The simulator does NOT draw these lines to final destination for sake of simplicity, but you can assume that all the RED lines in the diagram are controls. Some of these control are required for EXE step, other in MEM step and lastly very few are required for WB step.	Also note that the Immediate field is sign extended although it not required for ADD instruction but it still does not hurt. Note that only these values are displayed in HEX and all other values on the diagram appear in decimal.

Step 5: The Instruction goes Execution

Press clock it. In Exe step, the importance of each field of the instruction becomes quite clear.

Note 1: B input of the ALU is selected to be R3 register (whose value we initialized to 200) and 32-bit Sign extended Immediate field is rejected.

Note 2: The small control under ALU can still pick the right fields from immediate date and decodes this instruction to be ADD from the function code.

Note 3: At the top Branch target Address Adder assumes immediate field to be Branch Offset, multiplies it by 4 (by shifting left 2 times) and adds NPC. Verify that this is the answer that you got in the exercise 2.

Note 4. The main ALU adds R4 and R3 and AluOUT0 sets out to be 300.

Note 5: The bottom small multiplexer selects Rd from Rt and Rd as Rd is destination for this instruction. Hold your comments on this until you have seen the next instruction.

Step 6: The MEM stage.

This instruction has nothing to do with memory, so it bypasses the Data Memory in MEM stage. So simply Clock it through.

Note 1. AluOUT goes to memory but for non LW and SW instructions, memory simply returns 999 which, as you will see, is rejected in the WB stage. So there is nothing further for this instruction in this stage.

Step 7: The WB stage.

In the Write Back stage, first notice RegisterWrite is 1 for this instruction as this instruction is going to write in the Register File. Second notice that very last multiplexer selects AluOUT from AluOUT and Loaded Memory Data (LMD). Which in return is written to the register file at Rdest.

EXPERIMENT 2: Basic Instruction Processing the LW instruction:

STEP 1: Write single instruction program LW R7, - 200(R8), using the Integer Program Writer (IPW). You can use CLEAR to clear the program memory. Press Assemble and save it as singleLW just in case.

STEP2. The IF stage.

Press clock it and bring the instruction from Instruction memory. As before fill the table in exercise three.

STEP3. Now are before step through the instruction and fill the table.

Note 1. We read Rt from register file but it is not used as you will see when you step through the instruction. When asked for R8, enter 100010 and when asked for R7 just give a value of 7777.

Note 2: In EXE stage Rt is selected as destination. As there is no Rd for this instruction. Note 3: In EXE stage note what value is selected to be B input.

Note 4. In MEM stage, since this is memory related instruction, we are trying to load a value at address given by AluOUT, enter an initial value of this memory location when asked, say 525252.

Note 5. Note what is selected in WB stage.

EXPERIMENT 3: Basic Instruction Processing the SW instruction:

STEP 1: Write single instruction program SW R7, - 200(R8), using the Integer Program Writer (IPW). You can use CLEAR to clear the program memory. Press Assemble and save it as singleSW just in case.

STEP2. The IF stage.

Press clock it and bring the instruction from Instruction memory. As before fill the table in exercise 5.

STEP3. Now are before step through the instruction and fill the table.

Note 1. We read Rt from register file but it is not used as you will see when you step through the instruction. When asked for R8, enter 100010 and when asked for R7 just give a value of 7777.

Note 2: In EXE stage Rt is selected as destination. As there is no Rd for this instruction. Note 3: In EXE stage note what value is selected to be B input.

Note 4. In MEM stage, since this is memory related instruction, we are trying to store a value at address given by AluOUT, see the value that is stored in the memory pressing Memory Menu at the top. Note what is MEMWrite for this instruction.

Note 5. Note what is selected in WB stage. What is register write and why. Fill exercise 6.

EXPERIMENT 4: Basic Instruction Processing the ADDI instruction:

Write a single ADDI R5, R6, 1000 and run it through. Now you must be able to see the expected behavior this instruction in the Pipeline.

EXPERIMENT 5: The Pipelined processor: Using the IPW, write the following program: ADD R4, R5, R6

LW R7, 100(R8) SLTI R9, R10, -1000 SW R1, 200(R2) SUB R11, R12, R13

Assemble and save it as MyFirst. Clock it when asked for register value give related value, for example, for R3 give 33333, R4 Give 44444 and so on. For R10 and after give 10000, 11000, 12000 etc. For memory load give 77777 as initial value. Now clock this program and see how nicely each instruction is dealt on its merit in each stage of the pipeline. Fill the Exercise 7 when ADD instruction is in WB stage. . Notice that an

instruction becomes blue when it has completed. Make all of the above instructions blue and now fill final values of registers

EXPERIMENT 5: The Problems with Pipelined processor: Using the IPW, write the following program:

ADD R1, R2, R3 SUB R5, R6, R1 ADD R4, R1, R5 SW R1, 200(R4) LW R4, 100(R1)

Assemble and save it as MySecond.	Clock it when asked for register value again give related value, for example, for R3 give 33333, R4 Give 44444 and so on. Notice that an instruction becomes blue when it has completed. Make all of the above instructions blue and now fill exercise 8. What problems do you see?

EXPERIMENT 6: Inserting NOPs:

Using the IPW, verify which of the following program works properly. Notice that an Example 1.	ADD R1, R2, R3

SUB R5, R6, R1

Example 2.

Example 3.

Example 4.

ADD R1, R2, R3 NOP

SUB R5, R6, R1

ADD R1, R2, R3 NOP

NOP

SUB R5, R6, R1

ADD R1, R2, R3 NOP

NOP NOP

SUB R5, R6, R1

Now fill the table 9 for this exercise. As you see example 3 above works fine. Inserting these NOPs between data dependent instructions was done by hand in earlier computers and even today for processors with floating point unis built-in.

EXPERIMENT 7: Automatic NOPs --- STALL Dectection:

Before we get to a much better solution, let us bring Stall detection Hardware. This Hardware is not shown in the diagram but it is there and can be activated by selecting Stall Detection in Options menu. This compares Rs and Rt field in ID stage with Rdest in EXE, MEM, and stalls the pipeline if they are equal. By stalling it means, one copy of the instruction stays in the ID stage, but other copy goes through the pipeline as normal. Its RegWrite and MemWrite, however, are made to 0, so it does not changes the state of the processor. Reload MySecond, reset the processor and RUN the same program with Stall Detection ON and Fill Table 10. Note that it adds many ID stage stalls, solution next time.

Student ID 23281269, Section _____M/E.

Date:51-12-2005. Computer Archtecture LAB 2

Exercise 1:What is hex code for NOP or SLL R0, R0, R0 000000 00000 00000 00000 00000 000000

Exercise 2: By looking at the binary code of the instruction, discover the following fields:

Rs为4, Rt为3, Rd为5, 16-bit Imm (Imm16)不使用, Shift amount 00000, Function

Code 100000, Write MemWrite 0 and RegWrite 1 for this instruction. What is use of

16-bit Immediate field in this instruction? 对ADD(R-type)无用，只有I-type指令使用该字段

If the same instruction were a Branch instruction, the immediate field would have represented a Branch Offset. What is the potential Target Address 由0010 1000 0010 0000转为0x2820, 0x2820=2×4096+8×256+2×16+0=10272，立即数左移2位（乘4），指令地址按字对齐（每条指令占4字节）(PC+4) + 41088

Exercise 3: LW Instruction

Rs:8, Rt: 7, Rd不使用, 16-bit Imm (Imm16)0xFF38, Shift amount不使用, Function Code 不使用, Write MemWrite 0 and RegWrite 1 for this instruction. What is use of

Function filed and Shift Amount in this instruction? 在LW这种I型指令中，这两个字段没有作用，它们被合并为16位的立即数字段

If the same instruction were a Branch instruction, the immediate field would have represented a Branch Offset. What is the potential Target Address (PC+4)+imm<<2=(PC+4)-800

Exercise 4: Branch Target Found Adress(PC+4)-800, Is It same as EX.3 是的 B input to ALU -200, Why? 因为 LW 指令通过将基址寄存器 Rs (R8) 的内容与符号扩展的立即数相加来计算有效内存地址, ALUOut 99810 (因为R8 初始值为100010, 100010-200=99810), ALUOP = 加法, Why 通过加法来计算内存地址.

LMD 525252, Rdest 7 why this? 因为对于LW指令，Rt字段（R7）指定了加载数据的目标寄存器

RegWrite 1 , MemWrite 0 , What are two values at the input of very last MUX in the WB stage ALUOut (99810) 和 LMD (525252)， Which value is selected LMD (525252), why 因为 LW 指令的目的是将从内存加载的数据写回寄存器。MemtoReg 控制信号为1，选择LMD作为写回数据. Final Register File Values R7 = 525252, R8 = 100010

Exercise 5: SW instruction

Rs: 8, Rt 7, Rd 不使用, 16-bit Imm (Imm16) 0xFF38, Shift amount 不使用, Function Code 不使用, Write MemWrite 1 and RegWrite 0 for this instruction. What is use of Function filed and Shift Amount in this instruction? 对SW（I型）指令来说，funct和shamt字段没有用

If the same instruction were a Branch instruction, the immediate field would have represented a Branch Offset. What is the potential Target Address (PC+4)+imm<<2=(PC+4)-800

Exercise 6: Branch Target Found Adress (PC+4)-800, Is It same as EX.3 是的? B input to ALU -200, Why? I型Load/Store指令的第二个操作数来自立即数，而非寄存器Rt, ALUOut 99810, ALUOP = 99810, Why 因为要通过ALU加法来计算地址.

LMD 999, Rdest 7 why this? 模拟器显示Rx=7被传递到了WB级。这确认了Rt字段的值（寄存器7）被用作目标寄存器编号进行传递，但是最终不会写入

RegWrite 0, MemWrite 1, Why Store指令向内存写内容，不向寄存器写内容, What are two values at the input of very last MUX in the WB stage ALUOut出来的地址(99808，这里是99810为了和4字节对齐，所以变成99808), 存储的值(7777), What is selected ALUOut出来的地址(99808), why 因为RegWrite = 0，实际不会写回寄存器. Final Register File Values. R7 = 7777， R8 = 100010, 其他寄存器值不变

| Exercise 7: |  |  |
| --- | --- | --- |
| Signal Name | Value | Why |
| ID.A | 22222 |  |
| ID.B | 11111 |  |
| ALUOUT in EXE | 88988 |  |
| ALUOUT in MEM | 122221 |  |
| ALUOUT IN WB | 122221 |  |
| RDEST in ID | 1 | ID阶段的SW指令没有目标寄存器，但流水线将Rt字段(R1)的值向前传递。 |
| RDEST in EXE | 9 | EXE阶段SLTI指令的目标寄存器是 R9。 |
| RDEST in MEM | 7 | MEM阶段LW指令的目标寄存器是 R7。 |
| RDEST in WB | 4 | WB阶段ADD指令的目标寄存器是 R4。 |
| LMD in MEM | 77777 | LW指令正在从内存地址88988处读取数据。预设该地址的值为77777。 |
| LMD in WB | 999 | ADD指令不访问内存，因此传递到WB阶段的内存数据是一个默认/无效值(999) |
| Final Values of Register File |  |  |

Exercise 8: Expected Values of Register File

R1 11111

R2 22222

R4 122221

R5 55555

R6 66666

R7 77777

R8 88888

R9 0

R10 10000

R11 -1000

R12 12000

R13 13000

Actual Values of Register File

R5 11111 66666

Reason on difference

Exercise 9:

| Instruction\Clock Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD R1, R2, R3 | IF | ID | EXE | MEM | WB |  |  |  |  |  |  |  |
| NOP |  | IF | ID | EXE | MEM | WB |  |  |  |  |  |  |
| NOP |  |  | IF | ID | EXE | MEM | WB |  |  |  |  |  |
| SUB R5, R6, R1 |  |  |  | IF | ID | EXE | MEM | WB |  |  |  |  |

Exercise 10:

| Instruction\Clock Cycle | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD R1, R2, R3 | IF | ID | EXE | MEM | WB |  |  |  |  |  |  |  |
| SUB R5, R6, R1 |  | IF | ID | ID | ID | EXE | MEM | WB |  |  |  |  |
| ADD R4, R1, R5 |  |  | IF | IF | IF | ID | ID | ID | EXE | MEM | WB |  |
| SW R1, 200(R4) |  |  |  |  |  | IF | IF | IF | ID | ID | ID | EXE |
| LW R4, 100(R1) |  |  |  |  |  |  |  |  | IF | IF | IF | ID |

| Instruction\Clock Cycle | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ADD R1, R2, R3 |  |  |  |  |  |  |  |
| SUB R5, R6, R1 |  |  |  |  |  |  |  |
| ADD R4, R1, R5 |  |  |  |  |  |  |  |
| SW R1, 200(R4) | MEM | WB |  |  |  |  |  |
| LW R4, 100(R1) | ID | ID | EXE | MEM | WB |  |  |